BeMicro Max 10
FPGA Evaluation Kit

Getting Started User Guide

Version 14.0	9/4/2014	User Guide

Table of Contents

1.	Overview	2

1.1	Board Features	2

1.2	Block Diagram	3

1.3	Getting To Know Your Kit	4

2.	Software Installation	5

2.1	Install the Altera Design Software	5

2.1.1	Download and Install Quartus II Web Edition v14.0	5

2.1.2	Download and Install Update 2 which includes the MAX 10 FPGA device family support	11

2.2	Enable TalkBack	12

2.3	Install USB Blaster Driver	12

2.4	Download and Extract a BeMicro Max 10 Kit Example Project	15

3.	Pinout Information for MAX 10 FPGA I/O	16

3.1	Analog Devices External Peripherals	16

3.1.1	Accelerometer, 3-Axis, SPI interface (ADXL362)	16

3.1.2	DAC, 12-bit, SPI interface (AD5681)	16

3.1.3	Temperature sensor, I2C interface (ADT7420)	17

3.2	External Memory Devices	17

3.2.1	8MB SDRAM	17

3.2.2	Serial Flash	18

3.3	User Interaction	18

3.3.1	LEDs	18

3.3.2	Push Buttons	18

3.4	MAX 10 FPGA Analog Inputs	19

3.4.1	Analog Input Header	19

3.4.2	Photo Resistor	19

3.4.3	Thermistor (Thermal Resistor)	19

3.5	Expansion Headers and Connectors	20

3.5.1	BeMicro Edge Connector	20

3.5.2	Two 40-pin Expansion Headers	21

3.5.3	PMOD Connectors	22

3.6	Clock Inputs	22

3.7	Boot Select	22

4.	Hands-On Tutorials and Example Designs	23

LVDS loopback demo

The MAX 10 devices use registers and logic in the core fabric to implement LVDS input and output interfaces.

• For LVDS transmitters and receivers, MAX 10 devices use the the double data rate I/O (DDIO) registers that reside in the I/O elements (IOE). This architecture improves performance with regards to the receiver input skew margin (RSKM) or transmitter channel-to-channel skew (TCCS).

• For the LVDS serializer/deserializer (SERDES), MAX 10 devices use logic elements (LE) registers.

We set the demo for the LVDS loopback functions in the BeMicro Max 10.

The BeMicro Max 10 contains two 2x20 Pin headers(J4 and J5), many of them support LVDS, you can reference below list. we can use either of them for demo purpose.

| Signal Name; MAX 10 Pin; J3 Pin. DIFF_RX_P[0]; K14; 39. DIFF_RX_P[0](n); K15; 40. DIFF_RX_P[1]; E16; 37. DIFF_RX_P[1](n); E15; 38. DIFF_RX_P[2]; D17; 35. DIFF_RX_P[2](n); C17; 36. DIFF_RX_P[3]; H14; 33. DIFF_RX_P[3](n); J13; 34. DIFF_RX_P[4]; C14; 31. DIFF_RX_P[4](n); C13; 32. DIFF_RX_P[5]; A14; 27. DIFF_RX_P[5](n); B14; 28. DIFF_RX_P[6]; D14; 25. DIFF_RX_P[6](n); E13; 26. DIFF_RX_P[7]; E12; 23. DIFF_RX_P[7](n); D13; 24. DIFF_RX_P[8]; H12; 21. DIFF_RX_P[8](n); J11; 22. DIFF_RX_P[9]; B10; 19. DIFF_RX_P[9](n); C9; 20. DIFF_RX_P[10]; A9; 17. DIFF_RX_P[10](n); B8; 18. DIFF_RX_P[11]; A7; 15. DIFF_RX_P[11](n); A8; 16. | Signal Name; MAX 10 Pin; J4 Pin. LVDS_TX_P[0]; V17; 39. LVDS_TX_P[0](n); W17; 40. LVDS_TX_P[1]; V16; 37. LVDS_TX_P[1](n); U15; 38. LVDS_TX_P[2]; W15; 35. LVDS_TX_P[2](n); V14; 36. LVDS_TX_P[3]; W14; 31. LVDS_TX_P[3](n); V13; 32. LVDS_TX_P[4]; Y14; 29. LVDS_TX_P[4](n); Y13; 30. LVDS_TX_P[5]; AA10; 27. LVDS_TX_P[5](n); Y10; 28. LVDS_TX_P[6]; V10; 23. LVDS_TX_P[6](n); V9; 24. LVDS_TX_P[7]; AA7; 21. LVDS_TX_P[7](n); AA6; 22. LVDS_TX_P[8]; W8; 19. LVDS_TX_P[8](n); W7; 20. LVDS_TX_P[9]; U7; 15. LVDS_TX_P[9](n); U6; 16. LVDS_TX_P[10]; W6; 13. LVDS_TX_P[10](n); W5; 14. LVDS_TX_P[11]; W3; 11. LVDS_TX_P[11](n); W4; 12. |
| --- | --- |

LVDS loopback simple demo chart

Soft LVDS IP megawizard

Open Quartus-Tool-IP Catalog

Browse the Altera Soft LVDS IP

1. Altera Soft LVDS - lvds_tx IPcore setting - Page1

2. Altera Soft LVDS - lvds_tx IPcore setting - Page2

3. Altera Soft LVDS - lvds_tx IPcore setting - Page4

4. Altera Soft LVDS - lvds_rx IPcore setting - Page1

5. Altera Soft LVDS - lvds_rx IPcore setting - Page2

6.Altera Soft LVDS - lvds_rx IPcore setting - Page3

7.Altera PLL - Page1

8.Altera PLL - Page2

9.Finish the project looks like...

9.Connect the TX(P and N) with RX(P and N)

release all keys

push SW3

Push SW4

we can adjust the transmit data by below code

assign  tx_in_custom = 8'hA1;     //      Display constant

assign  tx_in_custom = counter;   //      Display Counter

assign  tx_in_custom = {6'b010100,key_rr};  //      Display the SW3 and SW4 key value

use the counter in the transmit data

use the constant 0xA1 in the transmit data